Hasil Penelitian	JurnalTeknol, dan Industri Pangan, Vol. XVII No. 2 Th. 2006

OPTIMASI PROSES MASERASI VANILI (Vanilla planifolia Andrews) HASIL MODIFIKASI PROSES KURING

[Maceration Process Optimation of Vanili (Vanilla Planifolia Andrews) from Modified Curing]

Dwi Setyaningsih1), Meika S, Rusli1), Melawati2) dan Ika Mariska3)

-Departemen Teknologi Industri Pertanian, Faleta, IPB

2)Alumni Departemen Teknologi Industri Pertanian, Fateta, IPB

3) Balai Penelitian Biologi dan Genetika Tanaman (Balit Biogen), Bogor

Diterima 15 September 2006/Disetujui 2 November 2006

ABSTRACT

Modified cured vanilla was processed to vanilla extract by maceration method. The aim of this research were to optimize the method of maceration, type of vanilla bean with highest vanillin content, extraction solvent composition,and other variables that could optimize the vanillin content and characterize the extract from half dried cured vanilla. The optimation used response surface method with 22 factorial and 23 factorial.

One step of maceration could extract vanillin (average 2.3 g/l) much more than two steps maceration (average 2.1 g/l). Vanillin content of the half dried cured vanilla (average 0.98 g/l) was higher than cured vanilla 1 and cured vanilla 2 (average 0.41 g/l and 0.32 g/l). The suitable ethanol-water composition forhalf dried cured vanilIa was 7:3 (vanillin content 1.78 g/l).

The first optimation was conducted with two variables maceration time and sucrose concentrations. The maximum vanillin content of the first optimation was 4.5 g/l at maceration time of 15.9 days and sucrose concentration of 7.3 g. The second optimation used two variables: maceration tima and glycerol concentrations. The maximum vanillin content of the second optimation was 3.8 g/l at maceration time of 22 days end glycerol concentration 19.9 ml. The third optimation process used three vanabies.maceration time, sucrose concentrations and glycerol concentrations. The maximum vanillin content of the third optimation was 3.4 gA at maceration time of 12 days sucrose concentration of 7 g, and glycerol concentration 4.7 ml. The characteristic of vanilla extract resulted from half dried cured vanilla maceration were vanillin content (3.4-4.5 g/l), total acid (380-410 ml 0.1 N NaOH/l), total ash (1.3-3.4 g/l), total soluble ash (0.8-2.9 g/l), alkalinity of total ash (462.6-536.7). alkalinity ofsoluble ash (139 1-216.5), and lead number (4.5-4 6).

Key wordi: Vanilla planifolia, Optimization, vanilla ekstrak